===================

Linux IOMMU Support

===================

The architecture spec can be obtained from the below location.

http://www.intel.com/content/dam/www/public/us/en/documents/product-specifications/vt-directed-io-spec.pdf

This guide gives a quick cheat sheet for some basic understanding.

Some Keywords

- DMAR - DMA remapping

- DRHD - DMA Remapping Hardware Unit Definition

- RMRR - Reserved memory Region Reporting Structure

- ZLR  - Zero length reads from PCI devices

- IOVA - IO Virtual address.

Basic stuff

-----------

ACPI enumerates and lists the different DMA engines in the platform, and

device scope relationships between PCI devices and which DMA engine  controls

them.

What is RMRR?

-------------

There are some devices the BIOS controls, for e.g USB devices to perform

PS2 emulation. The regions of memory used for these devices are marked

reserved in the e820 map. When we turn on DMA translation, DMA to those

regions will fail. Hence BIOS uses RMRR to specify these regions along with

devices that need to access these regions. OS is expected to setup

unity mappings for these regions for these devices to access these regions.

How is IOVA generated?

----------------------

Well behaved drivers call pci_map_*() calls before sending command to device

that needs to perform DMA. Once DMA is completed and mapping is no longer

required, device performs a pci_unmap_*() calls to unmap the region.

The Intel IOMMU driver allocates a virtual address per domain. Each PCIE

device has its own domain (hence protection). Devices under p2p bridges

share the virtual address with all devices under the p2p bridge due to

transaction id aliasing for p2p bridges.

IOVA generation is pretty generic. We used the same technique as vmalloc()

but these are not global address spaces, but separate for each domain.

Different DMA engines may support different number of domains.

We also allocate guard pages with each mapping, so we can attempt to catch

any overflow that might happen.

Graphics Problems?

------------------

If you encounter issues with graphics devices, you can try adding

option intel_iommu=igfx_off to turn off the integrated graphics engine.

If this fixes anything, please ensure you file a bug reporting the problem.

Some exceptions to IOVA

-----------------------

Interrupt ranges are not address translated, (0xfee00000 - 0xfeefffff).

The same is true for peer to peer transactions. Hence we reserve the

address from PCI MMIO ranges so they are not allocated for IOVA addresses.

Fault reporting

---------------

When errors are reported, the DMA engine signals via an interrupt. The fault

reason and device that caused it with fault reason is printed on console.

See below for sample.

Boot Message Sample

-------------------

Something like this gets printed indicating presence of DMAR tables

in ACPI.

ACPI: DMAR (v001 A M I  OEMDMAR  0x00000001 MSFT 0x00000097) @ 0x000000007f5b5ef0

When DMAR is being processed and initialized by ACPI, prints DMAR locations

and any RMRR's processed::

ACPI DMAR:Host address width 36

ACPI DMAR:DRHD (flags: 0x00000000)base: 0x00000000fed90000

ACPI DMAR:DRHD (flags: 0x00000000)base: 0x00000000fed91000

ACPI DMAR:DRHD (flags: 0x00000001)base: 0x00000000fed93000

ACPI DMAR:RMRR base: 0x00000000000ed000 end: 0x00000000000effff

ACPI DMAR:RMRR base: 0x000000007f600000 end: 0x000000007fffffff

When DMAR is enabled for use, you will notice..

PCI-DMA: Using DMAR IOMMU

Fault reporting

---------------

::

DMAR:[DMA Write] Request device [00:02.0] fault addr 6df084000

DMAR:[fault reason 05] PTE Write access is not set

DMAR:[DMA Write] Request device [00:02.0] fault addr 6df084000

DMAR:[fault reason 05] PTE Write access is not set

TBD

----

- For compatibility testing, could use unity map domain for all devices, just

provide a 1-1 for all useful memory under a single domain for all devices.

- API for paravirt ops for abstracting functionality for VMM folks.