virtio的eventfd机制浅析

virtio是KVM环境下的I/O虚拟化的框架，目前已经使用到的虚拟化设备包括block、net、scsi，本质是guest os与host os间的一种高效通信机制。本文旨在对virtio通信用到的eventfd进行分析。为了描述简单，一般来说，称guest侧通知host侧的行为为kick，host侧通知guset侧的行为为call。  参考资料：  1. virtio spec  2. Virtio 基本概念和设备操作  3. note  1. 什么是eventfd?  eventfd是只存在于内存中的文件，通过系统调用sys_eventfd可以创建新的文件，它可以用于线程间、进程间的通信，无论是内核态或用户态。其实现机制并不复杂，参考内核源码树的fs/eventfd.c文件，看数据结构struct eventfd_ctx的定义：   struct eventfd_ctx {          struct kref kref;          wait_queue_head_t wqh;          /*           * Every time that a write(2) is performed on an eventfd, the           * value of the __u64 being written is added to "count" and a           * wakeup is performed on "wqh". A read(2) will return the "count"           * value to userspace, and will reset "count" to zero. The kernel           * side eventfd_signal() also, adds to the "count" counter and           * issue a wakeup.           */          __u64 count;          unsigned int flags;  }; 1 2 3 4 5 6 7 8 9 10 11 12 13 14 1 2 3 4 5 6 7 8 9 10 11 12 13 14 eventfd的信号实际上就是上面的count，write的时候对其++，read的时候则清零（并不绝对正确）。wait_queue_head wqh则是用来保存监听eventfd的睡眠进程，每当有进程来epoll、select且此时不存在有效的信号(count <= 0)，则sleep在wqh上。当某一进程对该eventfd进行write的时候，则会唤醒wqh上的睡眠进程。代码细节参考eventfd_read(), eventfd_write()。  virtio用到的eventfd其实是kvm中的ioeventfd机制，是对eventfd的又一层封装（eventfd + iodevice）。该机制不进一步细说，下面结合virtio的kick操作使用来分析，包括两部分：  1. 如何设置：如何协商该eventfd？  2. 如何产生kick信号：是谁来负责写从而产生kick信号？  2. 如何设置？  即qemu用户态进程是如何和kvm.ko来协商使用哪一个eventfd来kick通信。  这里就要用到kvm的一个系统调用：ioctl(KVM_IOEVENTFD, struct kvm_ioeventfd)，找一下qemu代码中执行该系统调用的路径：  memory_region_transaction_commit() {     address_space_update_ioeventfds() {         address_space_add_del_ioeventfds() {             MEMORY_LISTENER_CALL(eventfd_add, Reverse, &section,             fd->match_data, fd->data, fd->e);         }     } } 1 2 3 4 5 6 7 8 1 2 3 4 5 6 7 8 上面的eventfd_add有两种可能的执行路径：  1. mmio(Memory mapping I/O): kvm_mem_ioeventfd_add()  2. pio(Port I/O): kvm_io_ioeventfd_add()  通过代码静态分析，上面的调用路径其实只找到了一半，接下来使用gdb来查看memory_region_transaction_commit()可能的执行路径，结合vhost_blk（qemu用户态新增的一项功能，跟qemu-virtio或dataplane在I/O链路上的层次类似）看一下设置eventfd的执行路径:  #0  memory_region_transaction_commit () at /home/gavin4code/qemu-2-1-2/memory.c:799 #1  0x0000000000462475 in memory_region_add_eventfd (mr=0x1256068, addr=16, size=2, match_data=true, data=0, e=0x1253760)     at /home/gavin4code/qemu-2-1-2/memory.c:1588 #2  0x00000000006d483e in virtio_pci_set_host_notifier_internal (proxy=0x1255820, n=0, assign=true, set_handler=false)     at hw/virtio/virtio-pci.c:200 #3  0x00000000006d6361 in virtio_pci_set_host_notifier (d=0x1255820, n=0, assign=true) at hw/virtio/virtio-pci.c:884 #4  0x00000000004adb90 in vhost_dev_enable_notifiers (hdev=0x12e6b30, vdev=0x12561f8)     at /home/gavin4code/qemu-2-1-2/hw/virtio/vhost.c:932 #5  0x00000000004764db in vhost_blk_start (vb=0x1256368) at /home/gavin4code/qemu-2-1-2/hw/block/vhost-blk.c:189 #6  0x00000000004740e5 in virtio_blk_handle_output (vdev=0x12561f8, vq=0x1253710)     at /home/gavin4code/qemu-2-1-2/hw/block/virtio-blk.c:456 #7  0x00000000004a729e in virtio_queue_notify_vq (vq=0x1253710) at /home/gavin4code/qemu-2-1-2/hw/virtio/virtio.c:774 #8  0x00000000004a9196 in virtio_queue_host_notifier_read (n=0x1253760) at /home/gavin4code/qemu-2-1-2/hw/virtio/virtio.c:1265 #9  0x000000000073d23e in qemu_iohandler_poll (pollfds=0x119e4c0, ret=1) at iohandler.c:143 #10 0x000000000073ce41 in main_loop_wait (nonblocking=0) at main-loop.c:485 #11 0x000000000055524a in main_loop () at vl.c:2031 #12 0x000000000055c407 in main (argc=48, argv=0x7ffff99985c8, envp=0x7ffff9998750) at vl.c:4592 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 整体执行路径还是很清晰的，vhost模块的vhost_dev_enable_notifiers()来告诉kvm需要用到的eventfd。  3. 如何产生kick信号？  大体上来说，guest os觉得有必要通知host对virtqueue上的请求进行处理，就会执行vp_notify()，相当于执行一次port I/O（或者mmio），虚拟机则会退出guest mode。这里假设使用的是intel的vmx，当检测到pio或者mmio会设置vmcs中的exit_reason，host内核态执行vmx_handle_eixt()，检测exit_reason并执行相应的handler函数（kernel_io()），整体的执行路径如下：  vmx_handle_eixt() {     /* kvm_vmx_exit_handlers[exit_reason](vcpu); */     handle_io() {         kvm_emulate_pio() {             kernel_io() {                 if (read) {                     kvm_io_bus_read() {                      }                 } else {                     kvm_io_bus_write() {                         ioeventfd_write();                 }             }         }     } } 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 最后会执行到ioeventfd_write()，这样就产生了一次kick信号。  如果该eventfd是由qemu侧来监听的，则会执行对应的qemu函数kvm_handle_io()；如果是vhost来监听的，则直接在vhost内核模块执行vhost->handle_kick()。  qemu kmv_handle_io()的调用栈如下所示:  Breakpoint 1, virtio_ioport_write (opaque=0x1606400, addr=18, val=0) at hw/virtio/virtio-pci.c:270 270   { (gdb) t [Current thread is 4 (Thread 0x414e7940 (LWP 29695))] (gdb) bt #0  virtio_ioport_write (opaque=0x1606400, addr=18, val=0) at hw/virtio/virtio-pci.c:270 #1  0x00000000006d4218 in virtio_pci_config_write (opaque=0x1606400, addr=18, val=0, size=1) at hw/virtio/virtio-pci.c:435 #2  0x000000000045c716 in memory_region_write_accessor (mr=0x1606c48, addr=18, value=0x414e6da8, size=1, shift=0, mask=255) at /home/gavin4code/qemu/memory.c:444 #3  0x000000000045c856 in access_with_adjusted_size (addr=18, value=0x414e6da8, size=1, access_size_min=1, access_size_max=4, access=0x45c689 <memory_region_write_accessor>, mr=0x1606c48)     at /home/gavin4code/qemu/memory.c:481 #4  0x000000000045f84f in memory_region_dispatch_write (mr=0x1606c48, addr=18, data=0, size=1) at /home/gavin4code/qemu/memory.c:1138 #5  0x00000000004630be in io_mem_write (mr=0x1606c48, addr=18, val=0, size=1) at /home/gavin4code/qemu/memory.c:1976 #6  0x000000000040f030 in address_space_rw (as=0xd05d00 <address_space_io>, addr=49170, buf=0x7f4994f6b000 "", len=1, is_write=true) at /home/gavin4code/qemu/exec.c:2114 #7  0x0000000000458f62 in kvm_handle_io (port=49170, data=0x7f4994f6b000, direction=1, size=1, count=1) at /home/gavin4code/qemu/kvm-all.c:1674 #8  0x00000000004594c6 in kvm_cpu_exec (cpu=0x157ec50) at /home/gavin4code/qemu/kvm-all.c:1811 #9  0x0000000000440364 in qemu_kvm_cpu_thread_fn (arg=0x157ec50) at /home/gavin4code/qemu/cpus.c:930 #10 0x0000003705e0677d in start_thread () from /lib64/libpthread.so.0 #11 0x00000037056d49ad in clone () from /lib64/libc.so.6 #12 0x0000000000000000 in ?? () 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19  1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 至此，整个virtio的evenfd机制分析结束。